Traffic Light Controller using FSM

LAB # 12

Spring 2024

CSE-308L Digital System Design Lab

Submitted by: Ali Asghar

Registration No.: 21PWCSE2059

Class Section: C

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work.”

Submitted to:

Dr. Asif Ali Khan

Date:

2nd June 2024

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

Objectives:

Implement a Traffic Light controller using FSM.

# Software used:

Xilinx ISE

Introduction

The lab this week will continue the introduction to FSMs with a simple Traffic Light Controller design project. In the lab, you are required to create a state diagram of a Mealy machine, which implements a traffic light controller, based on provided guidelines. You will then use this state diagram to write the behavioral Verilog description of the traffic light controller. The testing of your traffic light controller will take place during the lab session using the ISE development tools and the Spartan 6 board. As with the previous lab, we will make use of the character LEDs to display the outputs of our digital circuit.

The Traffic Light Controller

This week you will design a traffic light controller for the highway and farm road intersection shown in Figure 1. For simplicity, we will assume that the traffic lights on opposite ends of the intersection are the same; in other words, if the light for the North bound traffic is green, then the light for the South bound traffic is also green. The traffic light controller outputs two 2-bit signals, highwaySignal and farmSignal, for the highway road and the farm road, respectively.

Figure 1: Example Illustration

The following encoding is used for both signals:

001 : Green

010 : Yellow

100 : Red

Obviously, to avoid accidents, the highway lights and farm road lights must not be green at the same time. Furthermore, the transitions from green on each road must be followed by 3 seconds of yellow. Because the traffic on the highway is much greater than that of the farm road, the traffic light must remain green on the highway longer than on the farm road. For this design, the highway light must remain green unless there is a vehicle on the farm road, while the farm road light must remain green for only 10 seconds.

The brain of the traffic light controller is the FSM. For the initial design, Vehicle, Clock and Rst constitute the only input to the FSM. The output of the FSM includes the highwaySignal and farmSignal. The operation of the FSM came be described by using a state diagram as shown in figure 2.

Figure 1. Task1:

Task 2:

HG_FR

HR_FY

HR_FG

HY_FR

H=001 F=100

H=100 F=010

H=100 F=001

H=010 F=100

V = 0

V = 1

D < 3 sec

D = 3 sec

D < 10 sec

D = 10 sec

D < 3sec

D = 3 sec

Figure 2: State Transition Diagram

Lab Task 1:

Code:

Figure 1.1 top module

Figure 1.2: Clock Divider Code

Figure 1.3.1: Traffic Controller FSM Code – Part 1

Figure 1.3.2: Traffic Controller FSM Code – Part 2

Figure 1.4: Binary Coded Decimal (BCD) to Seven Segment Decoder

Figure 1.5: UCF File

Output:

Figure 1.6: RTL Schematic of Traffic Light Controller Top Module

Figure 1.7: Task 1 Implementation on Spartan6 FPGA Board (State S0)

Figure 1.8: Task 1 Implementation on Spartan6 FPGA Board (State S1)

Figure 1.9: Task 1 Implementation on Spartan6 FPGA Board (State S2)

Lab Task 2:

Code:

Figure 2.1 top module

Figure 2.2.1: Highway Traffic Controller FSM Code – Part 1

Figure 2.2.2: Highway Traffic Controller FSM Code – Part 2

Figure 2.2.3: Highway Traffic Controller FSM Code – Part 3

Figure 2.2.3: Highway Traffic Controller FSM Code – Part 4

Figure 2.3: UCF File

Output:

Figure 2.4: Task 2 Implementation on Spartan6 FPGA Board (State S0)

Figure 2.5: Task 2 Implementation on Spartan6 FPGA Board (State S1)

Figure 2.6: Task 2 Implementation on Spartan6 FPGA Board (State S2)

Figure 2.7: Task 2 Implementation on Spartan6 FPGA Board (State S3)