module ud_counter(

C,

R,

UD,  //high for UP counter and low for Down counter

COUNT

);

input C, R, UD;

output [3:0] COUNT;

reg [3:0] COUNT;

always @(posedge C or posedge R)

if(R)   //Active high R

COUNT <= 0;

else

if(UD)   //Up mode selected

COUNT <= COUNT + 1; //Increment Counter

else  //Down mode selected

COUNT <= COUNT - 1; //Decrement Counter

endmodule

module test_ud_counter;

reg clock;

reg reset;

reg mode;

wire [3:0] out;

ud_counter ud (.C(clock), .R(reset),

.COUNT(out), .UD(mode));

always

#5 clock = ~clock;

initial

begin

clock = 1'b0;

reset = 1'b1;

mode = 1'b0;

#7

reset = 1'b0;

#50

mode = 1'b1;

#80

mode = 1'b0;

#400 $finish;

end

initial

$monitor($time, "   UD = %b,   COUNT = %b", mode, out);

endmodule