module fsm_counter (count, clock);

input clock;

output [2:0] count;

reg [2:0] count;

//use parameter statement to define states

parameter A = 3'b000, B = 3'b100, C = 3'b111, D = 3'b010, E = 3'b011;

always @ (posedge clock)

//use case statement to implement state transitions

case(count)

A: count <= B;

B: count <= C;

C: count <= D;

D: count <= E;

E: count <= A;

default: count <= C;

endcase

endmodule

module test1_fsm_counter;

reg clock;

wire [2:0] count;

fsm_counter dut (count, clock);

initial

clock = 0;

always

#30 clock = ~clock;

endmodule

module test2_fsm_counter;

reg clock;

wire [2:0] count;

fsm_counter dut (count, clock);

initial

clock = 0;

always

#5 clock = ~clock;

initial begin

$display ("***********Time-----Output");

$monitor ($time, "   Count = %d", count);

end

endmodule