Name: ________________________

Registration: ___________________

Design and implement the following traffic light controller in Verilog.    (Simulation: 50% pts., FGPA Implementation: 50% pts.)

module traffic_lights2 (clk, light);

input clk;

output [2:0] light; reg [2:0] light;

parameter S0 = 0, S1 = 1, S2 = 2;

parameter RED = 3'b100, GREEN = 3'b010, YELLOW = 3'b001;

reg [0:1] state;

always @(posedge clk)

case (state)

S0: state <= S1;

S1: state <= S2;

S2: state <= S0;

default: state <= S0;

endcase

always @(state)

case (state)

S0: light = RED;

S1: light = YELLOW;

S2: light = GREEN;

default:	light = RED;

endcase

endmodule

`timescale 1s/1s

module tst_traffic_lights2;

reg clk;

wire [2:0] light;

traffic_lights2 tf (clk, light);

initial

clk = 0;

always

#5 clk = ~clk;

endmodule