Name: ________________________

Registration: ___________________

Design an 8x1 Mux using as building blocks the 2x1 and 4x1 Muxes shown below. Use the provided skeleton file to implement the 8x1 Mux.    (Simulation: 50% pts., FGPA Implementation: 50% pts.)

module m21 (OUT, A, B, S); input A, B, S; output OUT; assign OUT=S?B:A; endmodule

module m41 (OUT, A, B, C, D, S1, S0); input A, B, C, D, S1, S0; output OUT; reg OUT; always @(*) case ({S1, S0}) 0: OUT=A; 1: OUT=B; 2: OUT=C; 3: OUT=D; endcase endmodule

module m81 (OUT, I, Sel); input [7:0]; input [2:0] Sel; output OUT; //Write your code below endmodule

module m81 (OUT, I, Sel); input [7:0] I; input [2:0] Sel; output OUT; //Write your code below wire O1, O2; m41 m1 (O1, I[0], I[1], I[2], I[3], Sel[1], Sel[0]); m41 m2 (O2, I[4], I[5], I[6], I[7], Sel[1], Sel[0]); m21 m3 (OUT, O1, O2, Sel[2]); endmodule

module test_m81; reg [7:0] I; reg [2:0] Sel; wire OUT; m81 dut (OUT, I, Sel); initial begin I=8’b10110101; #10 Sel=3’d0; #10 Sel=3’d0; #10 Sel=3’d1; #10 Sel=3’d2; #10 Sel=3’d3; #10 Sel=3’d4; #10 Sel=3’d5; #10 Sel=3’d6; #10 Sel=3’d7; end endmodule